Synergy in ice clouds between airborne nadir pointing radar and lidar during CLARE’98

Anne GUYOT, Jacques TESTUD,

IPSL - Centre d'étude des Environnements Terrestre et Planétaires, Vélizy, France

Jacques PELON, Vincent TROUILLET

IPSL – Service d’Aéronomie, Paris, France

Samuel HAIMOV, David LEON, Gabor VALI

University of Wyoming, Laramie, WY, U.S.A.

# and Peter FRANCIS

UK Meteorological Office, Met. Research flights, Farnborough, U.K.

INTRODUCTION

A prototype airborne RAdar-LIdar (RALI) system was flown during the CLARE field project (in October 1998, near Chilbolton, UK). The RALI system currently under development at IPSL will consist of a 95 GHz cloud radar and a dual wavelength (0.5 and 10 µm) backscattering lidar on the same airborne platform. The test version consisted of the Service d'Aéronomie LEANDRE-1 lidar and the University of Wyoming cloud radar (Kestrel). These instruments were mounted on the IGN Fokker-27 aircraft (ARAT). The LEANDRE lidar had a fixed nadir looking beam, while Kestrel had a nadir beam and a beam 40 degrees forward of nadir.

The LEANDRE lidar is a 0.5 micron backscattering lidar with a 0.2 degree beamwidth, 10 ns pulse, 15 m range resolution (direct detection), and a 10 Hz repetition cycle. The Wyoming cloud radar is a 95 GHz Doppler radar. Kestrel beams both had a 0.7 degree beamwidth and 30 or 60 m range resolution. A 50 ns transmit/receiver switch network allowed pairs of pulses to be sent alternately to each antenna. Reflectivity and Doppler velocity were recorded for each beam.

During the CLARE campaign the U.K. Meteorological Office C-130 made some in-situ measurements. It flew simultaneously along the same legs as the ARAT, in clouds sampled by RALI. These measurements are since they allow to check the validity of our results for the analyzed CLARE data.

First, a method which combines radar reflectivity and lidar backscatter coefficient to infer properties of the particle size distribution is described. In this paper we focus on ice clouds. Then, the first results of the analysis applied to CLARE data are given.

SYNERGETIC ALGORITHM FOR RADAR AND LIDAR

In this section we present the mathematical formulation of a synergetic algorithm for radar and lidar. In a set of data, zones where both measurements of radar and lidar are available, and where there is only one type of particles will be selected for the analysis. The method is formulated for ice clouds.

In order to characterize the radiative and microphysical properties of a cloud, important parameters to determine are  ice water content IWC and effective radius re defined as:

IWC=π6ρiNDD3dD		        (1)

re=34ρiIWCα			 (2)

where N(D) is the droplet size distribution,  is the extinction coefficient, and i the density of solid ice (0.917.106 g/m3).

Considering LEANDRE wavelength (0.5 µm) and cloud particle sizes it can be assumed that D/ >> 1. This corresponds to the condition of the geometric approximation, where  is expressed as:

α=π4NDD2dD		(3)

At an attenuated frequency as 95 GHz, the radar does not measure the true reflectivity Z, but an apparent reflectivity Za subject to the two-way path attenuation. Za is related to Z through 		Za = Z – 2Kdr 		[dBZ]	       (4) where K (dB/km) is the specific attenuation.

For the formulation of the algorithm the assumption that a single type of particle is sampled is made. Then K and Z can be related with a powerlaw K = aZb. Under this assumption Hitschfeld and Bordan (1954) gave a solution of equation (4). By inverting the boundaries of the integral at the denominator, it can be expressed as:

Zr=Zar1+aIr,r01/b 		         (5)

where I(r,r0) = 0.46bZab(s)ds

Similarly, the assumption of a relation  = f is made (where  is the backscattering coefficient). It makes the constraining hypothesis that there is no molecular scattering. From Klett (1985) the attenuated backscattering coefficient can be expressed as a function of the extinction  and true backscattering coefficients:

βr=βar1+2fβasds		     (6)

From equations (5) and (6), specific attenuation K and extinction coefficient  profiles can be deduced:

Kr=Kr0ZabrZabr0+0.46bKr0Zabsds	       (7)

αr=αr0βarβar0+2αr0βasds		(8)

We set a relation between  and K ( = cKd), computed, as the K-Z relation, using a some Mie scattering calculations.

A constraint in the whole common zone of measurements: is chosen instead of a point by point constraint which would be to strong: αsds=cKsdds			       (9)

where r0 and r1 are respectively the lower and upper range of the common zone sampled by the two instruments.

Replacing  (7) and K (8) in (9), a relation (r0) = f((r0)) is obtained, where f((r0))is:

fαr0=βar02βasdsexp2αr0Jr1,r0−1    (10)

J(r1,r0)=ZarbZar0b+αr0c1/dIr,r0ddr

(r0) can be determined by resolving equation (10). This value is then replaced in (8) to obtain the (r) profile.

The same procedure is used for K(r) once K(r0) is estimated, using the  = cKd relation.

It is important to note the calibration of both instruments does not appear in the result of the algorithm since apparent reflectivities and backscattering coefficients are at numerator and denominator in each equation.

The last step is to determine the ice water content and effective radius profiles. For the first version of this method, relations between these parameters and K will be used to estimate the profiles. We use the specific attenuation instead of the true reflectivity in order to eliminate calibration problems.

Fig.1: 20th of October 14:41:51-14:48:01, upper part: Kestrel reflectivities along the leg, lower part LEANDRE attenuated backscattering coefficient.

ANALYSIS OF CLARE DATA

The only case of iced cloud measured by both instruments (below the ARAT) was sampled the 20th of October between 14:41:51 and 14:48:01, at the East side of the leg. Fig.1a and 1b display respectively reflectivity and attenuated backscatter coefficient measured along this leg. It is interesting to compare the two figures and see how the lidar penetrates in the ice cloud at 4 km altitude, while the signal is extinguished rapidly in all the other clouds sampled along the leg. In this study we focus on the part between –1.95° longitude and Chilbolton (14:45-14:48).

The first step is to determine the b coefficient of the K-Z relation, and the  = cKd relation. In order to do this, we make the hypothesis that the particle size distribution is a normalized gamma function:

ND=N03.67+µ43.674Γ4Γµ+4ΛDµexp−ΛD

The C-130 in-situ measurements were used to determine N0*.

Fig.2 displays N0* in the selected cloud. It appears that this parameter does not vary to much. It is chosen to be invariant in the relations we determine.

Fig.2: 20th Oct. 14:45-14:50, N0* computed from the C-130 2DC-2DP merged data

The K-Z relation b coefficient is computed from Mie scattering calculations, using a value of N0* of about 10^8.5. Whereas the K-re and K-IWC relations are directly computed from the in-situ measurements performed by the C-130.

Various profiles of effective radius and ice water content in the selected cloud are plotted in Fig.3. They were chosen to be representative of all the results we obtain.

These results can be compared to the horizontal profiles of ice water content and effective radius calculated from the C-130 microphysical data. The horizontal profiles (fig. 4) are at about 4.6 km, just above what was sampled by RALI, but they give a first guess of the values we should retrieve with our method.

In most of the cases, especially when there are at least 500 m of common measurements (longitude > -1.9°), IWC and re profiles are very similar from one profile to another. Ice water content increases from cloud base to the top of the measurements, where it reaches values close to 0.05 g/m3. These values seem to be underestimated when compared to the horizontal profile in fig. 4, IWC are about 2 or 4 times weaker than the estimation from in-situ measurements. Effective radius values at the top of the common zone are inconsistent with the estimation from 2DC-2DP data.

The profile at –1.911° is representative of all the profiles retrieved around and at the West of this longitude. It appears that IWC is overestimated and effective radius are too weak compared to the C-130 measurements computation. This problem in the retrieval is certainly due to the small thickness of the common zone of radar-lidar measurements.

CONCLUSION

During CLARE, the RALI instrument combination collected simultaneous measurements in clouds of a lidar nadir pointing and a radar nadir and 40° fore pointing.

When clouds are optically thin the lidar penetrates and makes simultaneous measurements with the radar. Then the synergetic algorithm formulated in this paper may be used. It allows to determine the parameters characterizing the cloud when there is only one type of particle sampled in the cloud.

Results obtained for the selected cloud during CLARE underestimate ice water content and effective radius, when referring to the C-130 in-situ measurements. It also clearly appears that when the common zone of radar and lidar measurements is thin, the retrieved profiles are not correct.

The method presented in this paper is a very first version which must be improved. A careful critical analysis of the analysis will be done in order to determine the origin of the error of estimation observed in the retrieval of IWC and re.

ACKNOWLEDGMENTS

The authors express their appreciation to K. Caillault (CETP) for providing her analysis of the microphysical data.

REFERENCES

Hitschfeld, W., and J. Bordan, 1954: Errors inherent in the radar measurement of rainfall at attenuating wavelengths. J. Meteor., 11, 58-67.

Klett, J. D., 1985: Lidar inversion with variable backscatter/extinction ratios. Applied Optics, 24, 1638-1643.

Fig.3: 20th Oct. (14:45-14:49) Retrieved ice water content IWC and effective radius re profiles for different longitudes using the synergetic radar-lidar algorithm, in the common zone of measurements.

Fig.4: 20th Oct. (14:41-14:49) IWC and re profiles along the leg computed from the C-130 in-situ measurements