Provision of Chilbolton Infrastructure and Instrumentation for CLARE 98.

J W F Goddard

- Radio Communications Research Unit

- Rutherford Appleton Laboratory, Chilton, Didcot, Oxon OX11 0QX, UK

##### Phone : +441235446127    Fax : +441235446140    Email: j.w.f.goddard@rl.ac.uk

# INTRODUCTION

The CLARE 98 campaign was centred on the Chilbolton Observatory in Hampshire, UK, which is operated by the Radio Communications Research Unit of the Rutherford Appleton Laboratory. Chilbolton not only provided several instruments, but also acted as a communications centre for controlling the entire operation and provided facilities for initial analysis of data from each day.  This document summarises the performance of the ground-based instruments operated by the Radio Communications Research Unit at Chilbolton during the CLARE campaign.

OVERVIEW OF CHILBOLTON FACILITIES

Chilbolton is a secure rural site ideally suited for mounting atmospheric science campaigns.  Figure 1 shows the 25 metre antenna with the main buildings in the background. The fully steerable 25 metre antenna is capable of supporting bolt-on experiments, and there is ample space for stand-alone equipment, either on the ground, on the flat roofs of the main buildings or in  portacabins. The Chilbolton intranet is connected to the main UK Janet network via a high speed telephone line, allowing fast internet access. Figure 2 shows how the various instruments were distributed around the site.

Fig. 1 The Chilbolton site, looking South East

Fig 2. Site plan of instrumentation at Chilbolton during the CLARE campaign (not to scale)

# RADARS

## 3 GHz CAMRa Radar

This radar uses the fully-steerable 25 m antenna (see Table 1 for details). Parameters measured and recorded during the campaign were reflectivity, differential reflectivity, linear depolarisation ratio, Doppler velocity and differential phase.

| Frequency | 3.075 GHz |
| --- | --- |
| Peak Power | 600 kW |
| Pulse Rep Frequency | 610 Hz |
| Pulse Width | 0.5 s |
| Antenna Diameter | 25 m |
| Beam Width | 0.26 |
| Polarisation | H/V |
| Scan Rate | 1 /sec |
| Unambiguous velocity |  15 m/sec |

Table 1: Details of the CAMRa radar

The radar was used to characterise the horizontal and vertical cloud and precipitation structure through regular RHI scans along the aircraft bearing, and, in addition, a special macro incorporating variable scan rates was used to allow the radar to track the aircraft flight. Vertical pointing data were not obtained, because the near-field distance of this radar is over 6 km, and also because ground clutter in the side/backlobes contaminates meteorological targets. Table 2 summarises the periods of data acquisition with the radar during CLARE. The CAMRa radar played an important role in calibration of the other radar systems used during CLARE.  For details, see Hogan and Goddard (1999).

| Day (Oct) | Scanning Reflectivity Data (UTC) |
| --- | --- |
| 7 | 0716 – 1500 |
| 8 |  |
| 9 | 1610 – 1812 |
| 10 |  |
| 11 |  |
| 12 | 1013 – 1045 1421 – 1526 |
| 13 | 0814 – 0937 1047 – 1615 |
| 14 | 1059 – 1503 |
| 15 | 1458 – 1548 |
| 16 | 1055 – 1335 |
| 17 |  |
| 18 |  |
| 19 | 0926 – 0940 1447 – 1708 |
| 20 | 0815 – 1601 |
| 21 | 0907 – 1104 |
| 22 | 0951 – 2044 |
| 23 |  |

Table 2: Summary of data from the 3 GHz CAMRa radar

## 35 GHz Rabelais Radar

This radar, on loan from the University of Toulouse, was mounted on the rim of the 25 m antenna, and so scanned in tandem with the CAMRa radar.

| Frequency | 34.94 GHz |
| --- | --- |
| Peak Power | 50 kW |
| Pulse Rep Frequency | 2688/3125 Hz |
| Pulse Width | 0.3 s |
| Antenna Diameter | 1.4 m |
| Beam Width | 0.43 |
| Polarisation | H/V |
| Scan Rate | 1 /sec |
| Unambiguous velocity |  6 m/sec |

Table 3: Details of the Rabelais radar

However, when scanning was not required, the radar was generally operated in a vertical pointing mode. Details of the radar are given in Table 3.  Parameters measured and recorded were reflectivity and differential reflectivity. Pulse-to-pulse data were also collected whenever aircraft flights were in progress. Table 4 summarises the periods of data collection with Rabelais. For details of the radar calibration see Hogan and Goddard (1999).

| Day (Oct) | Zenith Pointing Reflectivity Data (UTC) | Scanning Reflectivity Data (UTC) | Pulse-to-pulse IQ Data (UTC) |
| --- | --- | --- | --- |
| 7 |  | 0840 – 0910 1020 – 1030 1117 – 1500 | 1129 – 1501 |
| 8 |  |  |  |
| 9 | 1111 – 2320 | 1611 – 1820 |  |
| 10 | 0030 – 1630 |  |  |
| 11 |  |  |  |
| 12 | 1002 – 2319 |  |  |
| 13 | 0016 – 2302 | 1013 – 1547 | 1154 – 1548 |
| 14 | 0002 – 2350 | 1059 – 1503 | 1209 – 1503 |
| 15 | 0050 – 2325 |  |  |
| 16 | 1115 – 1545 | 0940 – 1507 | 1107 – 1458 |
| 17 | 0048 – 2349 |  |  |
| 18 | 0049 – 2350 |  |  |
| 19 | 0050 – 2318 |  |  |
| 20 | 0019 – 2306 | 0816 – 1539 | 0811 – 1528 |
| 21 | 0006 – 2358 | 0908 – 1104 | 0921 – 1123 |
| 22 | 0058 – 2306 | 0953 – 1047 1127 – 1436 | 1119 – 1510 |
| 23 | 0038 – 2351 |  |  |

Table 4: Summary of data from the 35 GHz Rabelais radar

### 95 GHz Galileo Radar

The hardware and software for this radar were still under development as the CLARE campaign got underway.  It was located in the Receive Cabin, operating through a window (reasonably transparent at 95 GHz) with an externally mounted flat reflecting plate at 45 to produce a vertically-pointing beam.  Details of the radar are given in Table 5.

| Frequency | 94.8 GHz |
| --- | --- |
| Peak Power | 2 kW |
| Pulse Rep Frequency | 6250 Hz |
| Pulse Width | 0.5 s |
| Antenna Diameter | 0.5 m |
| Beam Width | 0.5 |
| Unambiguous velocity |  5 m/sec |

Table 5: Details of the Galileo radar

Averaged reflectivity data were recorded towards the end of the campaign, using newly developed data acquisition software. Throughout the campaign, however, pulse-to-pulse data were collected whenever aircraft flights were in progress. Table 6 summarises the periods of data collection with the Galileo radar.  For details of the radar calibration see Hogan and Goddard (1999).

| Day (Oct) | Reflectivity Data (UTC) | Pulse-to-pulse IQ Data (UTC) |
| --- | --- | --- |
| 7 |  | 1127 – 1454 |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |
| 11 |  |  |
| 12 |  |  |
| 13 |  | 1322-1616 |
| 14 |  | 1205-1508 |
| 15 |  |  |
| 16 | 1115 – 1545 | 1058-1525 |
| 17 |  |  |
| 18 |  |  |
| 19 |  |  |
| 20 | 1350 – 1500 | 0844-1525 |
| 21 | 1005 – 1650 | 1026-1636 |
| 22 | 0925 - 1715 | 1119-1313 |
| 23 |  |  |

Table 6: Summary of data from the 94 GHz Galileo radar

#### LIDARS

IR Ceilometer CT75K

This instrument, manufactured by Vaisala and on long term loan from ESTEC, was located on the roof of the Control Room. Pointing at zenith, it recorded data continuously throughout the CLARE campaign. Details are shown in Table 7.

| Centre Wavelength | 905 nm |
| --- | --- |
| Peak Power | 16 W |
| Pulse Rep Frequency | 3670 Hz |
| Pulse Width | 100 ns |
| Resolution | 30 m |

Table 7: Details of the CT75K ceilometer

UV Raman Lidar

This lidar, although designed primarily for temperature and water vapour profiling, was operated throughout the campaign in a simple Rayleigh backscatter mode. Details are given in Table 8.

| Centre Wavelength | 354.7 nm |
| --- | --- |
| Peak Power | 1 MW |
| Pulse Rep Frequency | 50 Hz |
| Pulse Width | 7 ns |
| Resolution | 1 m |

Table 8: Details of the UV Raman Lidar

A full description of the lidar, together with analysis and comparisons with the IR ceilometer and the Galileo radar is given in Agnew et al, 1999.

| Day (Oct) | Zenith Pointing Data (UTC) |
| --- | --- |
| 7 | 1054 – 1630 |
| 8 | 1437 – 1600 |
| 9 | 0939 – 1000 1211 – 1310 1339 – 1400 |
| 10 |  |
| 11 |  |
| 12 | 0932 – 1237 |
| 13 | 1040 – 1555 |
| 14 | 1015 – 1515 |
| 15 | 1236 – 1315 1439 – 1458 |
| 16 | 1000 – 1525 |
| 17 |  |
| 18 |  |
| 19 | 1110 – 1209 1341 – 1543 |
| 20 | 0822 – 0913 1044 – 1540 |
| 21 |  |
| 22 | 1243 – 1514 |
| 23 |  |

Table 9: Summary of data from the

UV Raman Lidar

#### RADIOMETERS AND METEOROLOGICAL INSTRUMENTATION

### 93 GHz Radiometer

This instrument is operated at Chilbolton on behalf of the University of Bath. A failed pin switch on 13 October prevented further operation of the radiometer. Total liquid water estimates were made on three days,  7th, 12th and 13th October.

Meteorological Measurements

Routine meteorological measurements were made throughout the campaign, including rainfall, pressure, temperature and dew point. Finally, a cloud camera was installed at Chilbolton for the duration of CLARE. This recorded cloud images once every 15 seconds, along a typical aircraft bearing, covering a range of elevations from 45 to 90.

#### REFERENCES

[1] Hogan R. J. and Goddard J.W.F., “Calibration of the ground-based radars during CLARE 98”, This issue, 1999.

[2] Agnew J.L., Gibson A.J. and Marsh A.K.P., “Measurements for the CLARE 98 campaign using the Chilbolton UV lidar”, This issue, 1999.